OVERVIEW

The following describes how to verify the version on a programmed DECUS chip.

VERIFICATION

The DECUS version string is embedded in the program image.  To find the location of the version, compile the DECUS program (e.g., using the ”C” command) and then enter ”see .version.”  This will give you the start address of the version display routine.

A typical address would be $028D.  The first bytes at this location will be a call to the string display routine, ”string.”  Following the two call bytes is a count followed by the version characters.

For most MyForth projects, the version string is 8 characters so that it can be displayed on an LCD (if used for the project).  Thus, the version string always starts with ”08” followed by 8 ASCII characters.  Here is an example for the string ”DECUS 2 ” :

028D	51 82	acall 0282		\ call to string

028F	08							\ count

0290	44 45						\ DE

0292	43 55 53					\ CUS

0295	20 32 20					\ b2b