SOFTWARE NOTES

==============

When using a 5.25" disc in CP/M Plus or CP/M 2.2, the speed with

which the disc drive head is stepped must be reduced as a 3" drive

is faster than the 5.25" drive.   In CP/M 2.2, his can be done with

the SETUP.COM program on your system disc.  Changing the drive motor

step-time to 20 milli seconds should be sufficient.

With CP/M Plus however, a seperate program to slow the motor down

must be written.   The following small program does the trick.

;   SLOW DISC STEP RATE - Bob Francis 1987.

;

COMMON  EQU     0C300H

ORG     100H

LD      HL,(1)      ; Get BIOS warm start entry.

LD      DE,87       ; Offset to USERF function.

ADD     HL,DE       ; HL points to USERF f'n.

LD      (USER),HL   ; Store copy of USERF address.

LD      HL,BLOCK    ; Prepare to shift parameter

LD      DE,COMMON   ;   block to "common" memory area.

LD      BC,7

LDIR                ; Shift it.

LD      HL,COMMON

CALL    COPY        ; Reset disc controller.

RET                 ; Buggeroff back to CP/M...

COPY:   DEFB    0CDH        ; First byte of "CALL" inst.

USER:   DEFW    0           ; Addr of USERF f'n goes here.

DEFW    0083H       ; 83H = DD SETUP in extended BIOS.

RET                 ; To main routine...

BLOCK:  DEFB    10   ; Motor ON timeout * 100 ms.

DEFB    100  ; Motor OFF timeout * 100ms (default = 50).

DEFB    175  ; Write current OFF time * 10us.

DEFB    30   ; Head settle time * 1ms.

DEFB    20   ; Motor STEP RATE * 1ms (default = 12).

DEFB    0FH  ; Head unload delay * 32ms.

DEFB    03H  ; Head load delay and non-DMA set.

Just for completeness, here's a BASIC porgram that does the same

thing for when using the second 5.25" drive in AMSDOS:-

10 '<<<<< SLOW STEP RATE - R.D. Francis 1987 >>>>>

20 RESTORE 100

30 mem = HIMEM: MEMORY 34999

40 FOR i = 0 TO 45

50   READ c: POKE i+35000,c

60 NEXT i

70 CALL 35000

80 MEMORY mem

90 '

100 DATA 33,217,136,205,212,188,56

110 DATA 6,62,7,205,90,187,201,125

120 DATA 50,218,136,124,50,219,136

130 DATA 121,50,220,136,33,221,136

140 DATA 223,218,136,201,130,0,0,0

150 DATA 50,0,244,1,175,15,20,1,3

160 '    Step rate in ms = ^^

( Note that these programs have been read into this file using a

wordprocessor from original source code to avoid transcription errors ).

HARDWARE NOTES

==============

Power supply.

-------------

The second drive requires a seperate power supply.   This will need

to be approx 1 Amp at 5 volts and 0.5 Amp at 12 volts.   The standard

power supply connector for the drive looks as follows - (as seen

looking into the pins) .....

____________

|            |

| O  O  O  O |

|____________|

== == == ==

^  ^  ^  ^

5v.  \/   12v.

0v.

(Gnd)

Select switches.

----------------

The Drive will have a DIL switch or similar method of setting the

drive up as drive No.2.   This switch will (probably!) be marked:HL,

1,2,3,MX,4,and MH.   On my drive,I set the switches as follows:

1  = OFF   2  = ON   3  = OFF   4  = OFF

HL = OFF   MX = OFF  MH = ON

The READY signal.

-----------------

The READY signal on the disc interface must be pulled LOW (ie to

0 volts), whenever there is a disc in the second drive.   The easiest

way to arrange that is to fit a sub miniture, normally open micro-

switch to the disc drive door so it MAKES when the door is shut:

/

/

To 0 volts <---------O    O----------> to finger 34 on disc drive pcb

switch           edge connector.

The Connectors.

---------------

Probably the easiest way to fit a connector for the second drive

to a DD1's cable is to put an 34 way female IDC card edge connector

about two thirds of the way along it, (but make sure there is

sufficient length to place the two drives side by side!).

For reasons best known to themselves, Amstrad have their cable

"back-to-front", which means you may need to turn the connector over

by 180 degrees to get it to work!

To fit a second drive to a 6128/664 you simply need about half a

metre of 34 way ribbon cable with a 34 way female IDC card edge

connector at each end - turning it over as before.

Bob Francis. Sept '87.

#le with a 34 way female IDC card edge

connector at each end - turning it over as before.

Bob Francis. Se