CACHE.COM

A memory cache facility for the KAYPRO II.

By Steve McMahon

For the Bay Area Kaypro Users & Programmers.

February 26, 1983

INTRODUCTION

One of the oft-cited failings of CP/M is its lack of any

way to `redirect' output intended for a logical device such as

the list device or console to a disk file.

The uses of such a redirect feature would be manifold.

Transcripts of console operations could be kept, directory

reports could be included in text files, Word Processor output

could be saved in a disk file for special modification, etc.  The

ability to save the output of Perfect Printer in a disk file

would allow the insertion of special graphics characters

otherwise unavailable when redirection was used in conjunction

with a text editor.

A memory cache provides one way to get this type of

redirection. The idea is to reserve a chunk of RAM out of the way

of anything else and to make it into the logical LIST device.

All list output will then be stored in the cache.  Console output

may also be placed there by using a ^P to cause console output to

be echoed to the LIST device (the memory cache).  However, some

scheme must then be devised to get the cache into a disk file.

CACHE.COM does just these things.  The program overlays

the Basic Input Output System (BIOS) of a 56k version of KayPro

CP/M with a BIOS altered to create and control a cache in the top

8k of RAM.  (Using a 56k version of CP/M leaves this area

unused by the system, allowing its use for the cache.)

To use CACHE, you will have to use MOVCPM.COM and

SYSGEN.COM to create a 56k version of CP/M on disk.  The new

version of CP/M must actually be installed on disk.  Though

MOVCPM will allow you to create and immediately run a 56k version

without writing it onto a disk, this will not work with CACHE!

(Because when working this way MOVCPM is actually invoked at each

warm start to reinstall a whole new BIOS.  It overwrites the

cache BIOS.)

PUTTING A 56K VERSION OF CP/M ON A DISK

The following is a sample transcript of an installation

of a 56k version of CP/M on the disk on drive B:, using programs

on a disk in A:.  The disk in drive A: is unaffected, the B: disk

will become the cache operating disk.

-----------------Transcript of 56k installation----------------

A>

A>movcpm 56 *

CONSTRUCTING 56k CP/M vers 2.2

READY FOR "SYSGEN" OR

"SAVE 34 CPM56.COM"

Warm Boot

A>sysgen

KAYPRO SYSGEN VER 2.2

SOURCE DRIVE NAME (OR RETURN TO SKIP) {Enter a return here}

DESTINATION DRIVE NAME (OR RETURN TO REBOOT)b {enter b for B: drive}

DESTINATION ON B, THEN TYPE RETURN {Another return here}

FUNCTION COMPLETE

DESTINATION DRIVE NAME (OR RETURN TO REBOOT) {Yet another return here}

Warm Boot

A>

--------------------End Transcript------------------

You will probably also wish to put CACHE.COM on the 56k

CP/M disk.

USING CACHE.COM TO FILE LIST OUTPUT

The 56k CP/M disk must be placed in A: and cold booted

(push the reset button).  The cache may then be installed as

simply as typing CACHE {RETURN}.  The program will notify you if

the overlay is successful.  As a reminder of the modified BIOS's

presence, warm boots will be signalled by "Reloading" rather than

"Warm Boot".

Now, any output directed to the LIST device will instead

go to the cache.  Typing ^P at the console will toggle on and off

the LIST echoing of what goes on the CRT screen -- whatever

happens between ^P's will be saved in the cache (It will not go

to the printer!).

If you over-run the 8k of buffer, the console bell will

sound and the cache will reset itself.  Earlier cache contents

will be lost, but nothing dreadful happens to the operating

system or whatever your working on.

The cache resets itself with each warm boot.  On

receiving a warm boot command (such as a ^C or the end of a

program), the cache BIOS will move the memory cache to the

Transient Program Area (TPA) beginning at 100H and will indicate

in pages of 256 bytes (100H) how much has been saved.

The output may now be saved in a disk file with the CP/M

resident command SAVE.  Note that SAVE expects the number of

pages to be saved in DECIMAL, while the console report is in

HEXADECIMAL.  (Sorry about that, I was too lazy to write a

conversion routine.)

So, if you received the report:

--------------------------------------------

Reloading

0DH pages saved beginning at 100H

A>

--------------------------------------------

and wished to store the fruits of your labor in a file

B:MYDOC.FIL, you should enter the command:

SAVE 13 B:MYDOC.FIL

(Note that 13 is the decimal form of 0DH.  If base

conversion does not come easy to you, there are tables available

to make the conversion.)

NOTE THAT ONCE A WARM BOOT HAS OCCURRED AND A SAVE NOTE HAS BEEN

RECEIVED AT THE CONSOLE, RUNNING ANY TRANSIENT CP/M PROGRAM

BEFORE SAVING TO A DISK FILE WILL CAUSE THE LOSS OF WHAT HAS BEEN

SAVED IN RAM.  (Note also that this is an exaggeration: if you

know how to use DDT you may recover the actual cache from 0DE00H

on, move it to the TPA and save it.  There are nearly always ways

to recover if you know what to do.)

Parts of CACHE are loosely derived from programs

described in Alan R. Miller's MASTERING CPM (SYBEX, 1983).

Steve McMahon

Berkeley

#f y